| AEPL | STANDARD OPERATING PROCEDURE CONTINUOUS FOTA AUTOMATION UTILITY | SOP No: AEPL/SOP/QA/FOTA-001 Rev No: 2.0.0 |
| --- | --- | --- |
| DEPT: QA & HARDWARE TESTING | EFFECTIVE DATE: 24-08-2026 | REVIEW DATE: 24-08-2027 |
| TARGET HARDWARE: Accolade 4G TCU | DOCUMENT TITLE: FOTA Utility SOP | AUTHOR: QA Automation Team |

size: A4 portrait;

margin: 15mm 12mm 15mm 12mm;

@bottom-right {

content: "Page " counter(page) " of " counter(pages);

font-size: 8pt;

color: #475569;

font-family: -apple-system, BlinkMacSystemFont, "Segoe UI", Roboto, Helvetica, Arial, sans-serif;

color: #0f172a;

line-height: 1.45;

font-size: 9.5pt;

h1, h2, h3, h4 {

color: #0f172a;

font-weight: 700;

width: 100%;

border-collapse: collapse;

margin-top: 6px;

margin-bottom: 12px;

font-size: 8.5pt;

background-color: #0f172a;

color: #ffffff;

font-weight: 700;

text-align: left;

padding: 6px 8px;

border: 1px solid #0f172a;

padding: 5px 8px;

border: 1px solid #cbd5e1;

vertical-align: top;

tr:nth-child(even) td {

background-color: #f8fafc;

.sop-header-table th, .sop-header-table td {

border: 1px solid #0f172a !important;

font-family: "Consolas", "Courier New", monospace;

background-color: #f1f5f9;

color: #0f172a;

padding: 1px 4px;

border-radius: 3px;

font-size: 8pt;

background-color: #0f172a;

color: #f8fafc;

padding: 10px;

border-radius: 4px;

font-family: "Consolas", "Courier New", monospace;

font-size: 8pt;

line-height: 1.35;

overflow-x: auto;

white-space: pre-wrap;

word-wrap: break-word;

margin-top: 6px;

margin-bottom: 12px;

background-color: #eff6ff;

border-left: 4px solid #2563eb;

padding: 8px 12px;

margin: 10px 0;

font-size: 8.5pt;

background-color: #fffbeb;

border-left: 4px solid #d97706;

padding: 8px 12px;

margin: 10px 0;

font-size: 8.5pt;

background-color: #fef2f2;

border-left: 4px solid #dc2626;

padding: 8px 12px;

margin: 10px 0;

font-size: 8.5pt;

break-after: page;

AEPL

STANDARD OPERATING PROCEDURE

SOP No: AEPL/SOP/QA/FOTA-001

Rev No: 2.0.0

CONTINUOUS FOTA AUTOMATION UTILITY — USER & OPERATOR MANUAL

Effective Date: 24-08-2026

Review Date: 24-08-2027

DEPARTMENT: QUALITY ASSURANCE & TESTING ENGINEERING

Target Device: Accolade 4G TCU (AIS-140)

## 1.0 UTILITY OVERVIEW & PURPOSE

### 1.1 What is the FOTA Automation Utility?

The Continuous FOTA Automation Utility is a desktop application designed for testing and validating Firmware Over-The-Air (FOTA) updates on Accolade 4G Telematics / TCU Units (AIS-140 compliant).

Instead of manually typing commands, monitoring terminal logs for hours, and tracking firmware updates by hand, this utility connects directly to your device via a USB-to-Serial cable and automates the entire process:

Captures Device Telemetry: Automatically reads IMEI, UIN, VIN, ICCID, active state, and current firmware version from boot logs.

Automates FOTA Upgrades: Resolves firmware versions, triggers upgrade jobs on the server, and tracks download progress in real-time.

10-Stage Progression Bar: Shows a visual 10-card pipeline (S1 through S10) indicating exactly which stage the device is in.

Verifies State & IP Configurations: Checks whether state server settings (*GET#SWEMP#, *GET#CHTP#, *GET#CIP1#) match target server matrix rules.

Logs & Exports Results: Records full audit logs, login packet histories, and enables single-click Export CSV reporting.

---

## 2.0 PREREQUISITES & PRE-START HARDWARE SETUP

Before launching the utility, ensure your test bench hardware is physically connected and powered correctly.

### 2.1 Required Equipment

Workstation PC: Windows 10 or 11 (64-bit).

Target Device: Accolade 4G TCU Unit (AIS-140 Compliant).

Power Supply: DC Power Supply (12V or 24V DC, minimum 2A output).

USB-to-Serial Converter Cable: FTDI, Prolific, or CH340 USB-Serial Cable connected to the TCU debug serial port (TX/RX/GND).

SIM Card & Antennas: Active cellular SIM card inserted, with 4G LTE and GNSS (GPS) antennas connected.

┌─────────────────┐       +12V / GND Power      ┌──────────────────────────────┐
 │ Regulated DC    ├────────────────────────────►│ Accolade 4G TCU Unit (DUT)   │
 │ Power Supply    │                             │ (AIS-140 Compliant)          │
 └─────────────────┘                             └──────────────┬───────────────┘
                                                                │ Serial Debug Cable
                                                                ▼ (TX / RX / GND)
 ┌─────────────────┐       USB Connection        ┌──────────────┴───────────────┐
 │ Workstation PC  │◄────────────────────────────┤ USB-to-Serial Adapter Cable  │
 │ (FOTA Utility)  │                             │ (FTDI / CH340 / Prolific)    │
 └─────────────────┘                             └──────────────────────────────┘

HARDWARE HANDLING PRECAUTIONS:

ESD Protection: Wear an Anti-Static Electrostatic Discharge (ESD) wrist strap when touching bare TCU circuit boards.

Power Supply Polarity: Verify +12V/24V (Red) and GND (Black) before turning on the power supply.

Close Other Serial Terminals: Close TeraTerm, PuTTY, RealTerm, or Arduino Serial Monitor before opening the FOTA Utility.

---

## 3.0 CONFIGURATION FILES & INITIAL SETUP

The utility uses two configuration files located in your application folder: .env and input/servers.json.

### 3.1 `.env` File (Application Credentials & Settings)

Located in the main application root folder. Ensure the following parameters are configured:

# Portal Credentials & Auth
PORTAL_LOGIN_URL=https://aepl-tcu4g-qa.accoladeelectronics.com:6101/api/user/login
PORTAL_USER=suraj.bhalerao@accoladeelectronics.com
PORTAL_PASS=your_password
USER_ID=677cd2181b05f196abd64907

# Serial Communication & Default State
SERIAL_PORT=COM5
SERIAL_BAUD=115200
DEFAULT_STATE=Continues Fota

# Accolade FOTA REST API Endpoints
FETCH_SERVERS_API_URL=https://aepl-tcu4g-qa.accoladeelectronics.com:6101/api/server/getServerData?page=1&size=50&search=
FETCH_SERVER_DATA_BY_ID=https://aepl-tcu4g-qa.accoladeelectronics.com:6101/api/server/getServerDataByUId?id={id}
FETCH_FOTA_HISTORY_URL=https://aepl-tcu4g-qa.accoladeelectronics.com:6101/api/fota/getFOTADevicesHistory?imei={imei}
FOTA_TRIGGER_API_URL=https://aepl-tcu4g-qa.accoladeelectronics.com:6101/api/fota/createManualFota

### 3.2 `input/servers.json` File (State Server Matrix)

Located in the input/ folder. This file contains server IP addresses, state abbreviations, state OTA enable strings, and available firmware versions per state:

[
  {
    "state": "Maharashtra",
    "stateAbbreviation": "MH",
    "govtIp1": "103.211.218.66",
    "govtPort1": "8086",
    "govtIp2": "103.211.218.67",
    "govtPort2": "8086",
    "stateEnable": "STATUS#SET#SWEMP#1#OK#",
    "firmwares": ["5.2.8_REL25", "5.2.9_REL25", "5.2.12_REL25"]
]

Automatic Matrix Sync & PyInstaller Fallback: The utility automatically syncs input/servers.json with the live server API whenever the application launches. When running as a standalone executable (Continuos_Fota.exe), if input/servers.json is missing or empty, the utility automatically extracts the bundled fallback matrix from PyInstaller resources on startup.

---

## 4.0 USER INTERFACE (UI) & OPERATOR CONTROL GUIDE

The user interface is designed for high visibility and single-click operation.

┌──────────────────────────────────────────────────────────────────────────────────────────────────────────┐
│ FOTA UTILITY  ● ONLINE (COM5 @ 115200)             Target State: [ Maharashtra ▾ ]  Port: [ COM5 ▾ ] [Refresh] [Theme] [Stop Engine] │
├──────────────────────────────────────────────────────────────────────────────────────────────────────────┤
│ [ 🖥️ Live Serial Console ]   [ 📊 Audit Log History ]   [ 📈 Analytics & Reporting ]   [ 📦 Login Packets ]        │
├──────────────────────────────────────────────────────────────────────────────────────────────────────────┤
│ IMEI: 861564069210428 | UIN: ACON4NA... | VIN: ACC... | ICCID: 899... | STATE: Maharashtra | FW: 5.2.9   │
├──────────────────────────────────────────────────────────────────────────────────────────────────────────┤
│ FOTA Download Progress: 100.00% [======================================================================] │
├──────────────────────────────────────────────────────────────────────────────────────────────────────────┤
│ [S1: Telemetry] [S2: Matrix] [S3: Sync] [S4: Audit] [S5: 100%] [S6: Reboot] [S7: State] [S8: IP1] [S9: IP2] [S10: Config] │
│   ✓ PASSED       ✓ PASSED     ✓ PASSED   ✓ PASSED   ✓ PASSED   ✓ PASSED     ✓ PASSED   ✓ PASSED  ✓ PASSED   ✓ PASSED  │
├──────────────────────────────────────────────────────────────────────────────────────────────────────────┤
│ ℹ️ STATUS | FOTA Downloading: 100.00% | Status: Completed                                                  │
└──────────────────────────────────────────────────────────────────────────────────────────────────────────┘

### 4.1 Header Bar & Port Selection Controls

Target State Dropdown (`Target State:`): Selects the target state server (*Maharashtra*, *Bihar*, *Assam*, *Default*).

Port Dropdown (`Port:`): Lists all connected workstation COM ports with hardware classification:

COM5 [Serial] - USB Serial Port [VALID PHYSICAL SERIAL PORT]

COM4 [Bluetooth] - Standard Serial over Bluetooth [DISALLOWED BLUETOOTH PORT]

Refresh Button (`Refresh`): Scans workstation USB ports quietly without triggering popup errors (Ctrl+R).

Theme Button (`Theme: Light` / `Theme: Dark`): Toggles visual dark and light UI color schemes (Ctrl+T).

Engine Button (`Start Engine` / `Stop Engine`): Starts or stops serial log reading and FOTA monitoring.

BLUETOOTH PORT GUARD: Selecting a Bluetooth port (e.g. COM4 [Bluetooth]) displays a warning alert and blocks starting the engine. Always select a physical USB-to-Serial COM port (e.g. COM5 [Serial]).

### 4.2 Telemetry Summary Card

Displays live hardware information extracted automatically from boot logs:

IMEI: 15-digit cellular module serial number.

UIN: Accolade unit identification number (ACON...).

VIN: Vehicle identification number.

ICCID: SIM card identification number.

CURRENT STATE: Target server state active on the unit.

FIRMWARE: Current active firmware version (e.g. 5.2.9 5th IP).

Clear Info Button: Clears the displayed telemetry summary fields.

### 4.3 Visual Download Progress Bar

Shows live download percentage (0.00% to 100.00%) with 2-decimal precision.

Updates in real-time as [FOT] downloading XX.XX% log lines are received from the device.

---

### 4.4 10-Stage Progression Bar (Cards Guide)

The 10-Stage Progression Widget (`StageProgressionWidget`) displays 10 visual cards showing the status of each validation stage:

| Card # | Stage Name | What Happens in This Stage | Badge Statuses & What They Mean |
| --- | --- | --- | --- |
| S1 | S1: Telemetry | Reads UIN, IMEI, VIN, Version, and State from serial boot logs. Saves pre-upgrade snapshot. | PASSED: Telemetry captured.WAITING: Waiting for boot logs. |
| S2 | S2: Servers Matrix | Verifies if current firmware version and state exist in input/servers.json. | PASSED: Target found in matrix.FAILED: Version not in matrix. |
| S3 | S3: Progress Sync | Checks server history for existing FOTA jobs. | PASSED: Sync complete.RUNNING: Attaching to active run. |
| S4 | S4: Audit Report | Creates run record in CSV, JSON, and session log files. | PASSED: Audit record created. |
| S5 | S5: 100% Download | Tracks FOTA download progress until it reaches 100.00%. | PASSED: Download complete.RUNNING: Download in progress. |
| S6 | S6: Device Reboot | Monitors post-download device reset log lines (GSM soft shutdown pass, MQTT disconnect). | PASSED: Device rebooted.RUNNING: Waiting for reboot. |
| S7 | S7: State OTA Fired | Issues *GET#SWEMP#. Verifies if state OTA is enabled on device. | PASSED: State verified.ALREADY SET: Pre-configured.NOT PRESENT: Parameter missing. |
| S8 | S8: IP1 & Port Set | Issues *GET#CHTP#. Verifies primary server IP and port configuration. | PASSED: IP1 verified.ALREADY SET: Pre-configured.NOT PRESENT: Parameter missing. |
| S9 | S9: IP2 & Port Set | Issues *GET#CIP1#. Verifies secondary server IP and port configuration. | PASSED: IP2 verified.ALREADY SET: Pre-configured.NOT PRESENT: Parameter missing. |
| S10 | S10: Config Verified | Verifies post-upgrade version from 55AA Login Packet matches target version. | PASSED: Version verified.FAILED: Version mismatch. |

UNDERSTANDING STAGE BADGES & ENFORCEMENT RULES:

PASSED: Stage completed successfully.

ALREADY SET: Device is already configured with target state/IP; stage passed automatically.

NOT PRESENT: Parameter absent in servers.json matrix; stage skipped safely.

FAILED: Stage validation failed (check Troubleshooting guide).

CRITICAL VALIDATION & RESET LAWS:

Strict Download Barrier: Stages S6 through S10 will NEVER execute or mark as passed until Stage S5 (100% Downloaded) reaches exactly 100.0%.

Re-download & Pre-abort Reset Rule: If the device's FOTA attempt count increases on the server, or a pre-abort handshake (STATUS#CLR#FOTA#OK / $CLR,FOTA,OK) is received, or live download progress restarts below 100%, Stages S5 through S10 automatically reset to [⏱ WAITING].

---

## 5.0 TAB-BY-TAB USER DASHBOARDS GUIDE

### 5.1 Tab 1: Live Serial Console & Control

Monospace Log Terminal: Displays live serial boot logs in white text on obsidian dark background.

Interactive Command Bar: Allows sending custom serial commands (*SET#CRST#1#, *GET#CHTP#) directly to the device. Press Up/Down arrow keys to recall previous commands.

Clear Buttons:

Clear Info: Resets telemetry summary fields.

Clear Log: Clears terminal log console (Ctrl+K).

### 5.2 Tab 2: Audit Log History

4 Metric Cards: Displays TOTAL EXECUTIONS, COMPLETED, ABORTED / FAILED, and SUCCESS RATE %.

Search & Status Filter: Filter audit records by IMEI, UIN, or Status (ALL, COMPLETED, ABORTED, IN_PROGRESS).

Export CSV Button: Single-click export saving audit logs to results/fota_results.csv.

### 5.3 Tab 3: Executive Analytics & Reporting

State Server Distribution Matrix: Shows total runs, passed runs, and pass rate % per state server.

Firmware Version Progression Table: Tracks firmware upgrade transitions and attempt history.

Export CSV Button: Exports combined analytics report to results/fota_analytics_report.csv.

### 5.4 Tab 4: Login Packets

Real-Time Login Capture: Automatically logs incoming 55AA GSM login packets with millisecond timestamps, IMEI, UIN, VIN, and Firmware Version.

Export CSV Button: Exports login packet history to results/login_packets.csv.

---

## 6.0 STEP-BY-STEP OPERATING WORKFLOW

Follow these simple steps to perform a complete FOTA validation test cycle:

STEP 1: Connect Hardware & DC Power Supply (12V/24V)
    │
    ▼
  STEP 2: Launch Utility Application (Continuos_Fota.exe)
    │
    ▼
  STEP 3: Select Target State (e.g. Maharashtra) & Physical Serial Port (COM5 [Serial])
    │
    ▼
  STEP 4: Click "Start Engine"
    │     (App locks port dropdown & automatically sends soft reboot *SET#CRST#1#)
    ▼
  STEP 5: Monitor 10-Stage Progression Bar (S1 ➔ S10)
    │
    ▼
  STEP 6: Verify Stage S10 (Config Verified) & View Audit History
    │
    ▼
  STEP 7: Click "Export CSV" to Save Official Test Report

### Detailed Operator Instructions

Connect Device: Connect TCU to DC power supply (12V/24V) and plug USB-Serial cable into workstation.

Launch Application: Double-click Continuos_Fota.exe (or run python main.py).

Select State: Pick your target state in the Target State Dropdown (e.g. *Maharashtra*).

Select Port: Click the Port Dropdown and select a physical USB Serial port (e.g. COM5 [Serial] - USB Serial Port). Do not select Bluetooth ports.

Start Engine: Click Start Engine. The utility connects, locks the port dropdown, and automatically fires soft reboot command *SET#CRST#1#.

Observe Progression: Watch boot telemetry populate in the top card and track cards S1 through S10.

Export Report: After S10: Config Verified passes, switch to Tab 2 or Tab 3 and click Export CSV to save your test records.

---

## 7.0 HARDWARE SERIAL COMMAND QUICK REFERENCE

| Command | Action / Description | Expected Device Response |
| --- | --- | --- |
| *SET#CRST#1# | Soft reboots the TCU hardware unit. | STATUS#SET#CRST#1#OK# / GSM soft shutdown pass |
| *GET#SWEMP# | Queries active state OTA status. | STATUS#SWEMP#<state>#OK# |
| *GET#CHTP# | Queries primary server IP1 and Port1. | STATUS#CHTP#<ip>:<port>#OK# |
| *GET#CIP1# | Queries secondary server IP2 and Port2. | STATUS#CIP1#<ip>:<port>#OK# |
| *CLR#FOTA# | Clears active FOTA update state on device. | STATUS#CLR#FOTA#OK# |

---

## 8.0 COMPLETE TROUBLESHOOTING & SELF-HELP GUIDE

If you encounter any issue or get stuck while using the utility, locate your symptom below to resolve it quickly:

| Problem / Error Message | What Caused It | How to Fix It (Step-by-Step) |
| --- | --- | --- |
| "Cannot Start Engine: COM4 is a Bluetooth port" | A Bluetooth serial link was selected in the Port dropdown. | 1. Open Port Dropdown.2. Select a physical USB Serial port (e.g. COM5 [Serial]).3. Click Start Engine. |
| "COM Port Access Denied" or Serial Connection Error | The COM port is occupied by TeraTerm, PuTTY, or another app. | 1. Close TeraTerm / PuTTY / Arduino IDE.2. Unplug and re-plug USB serial cable.3. Click Refresh (Ctrl+R) and click Start Engine. |
| "No COM Ports Found" in Dropdown | Workstation cannot detect USB-Serial adapter cable. | 1. Verify USB cable is firmly plugged in.2. Ensure FTDI / CH340 device drivers are installed in Windows Device Manager.3. Click Refresh (Ctrl+R). |
| "WARNING: Version X.X.X not found in servers.json" | Active firmware version on device is not listed in server matrix. | 1. Check Target State Dropdown to ensure correct state is selected.2. Open input/servers.json and add missing firmware version to firmwares list. |
| "API HTTP 401 Unauthorized Error" | API username or password in .env file is incorrect or expired. | 1. Open .env file in Notepad.2. Update API_USER and API_PASS with valid credentials.3. Restart the FOTA Utility. |
| Device Reboot Not Detected (Stage S6 Timeout) | Power supply turned off or device failed to soft shutdown. | 1. Verify DC Power Supply is outputting 12V/24V DC.2. Type *SET#CRST#1# in the bottom Command Bar and press Enter. |
| Stage S7 / S8 / S9 Shows "NOT PRESENT" | State parameter missing in input/servers.json for selected state. | 1. Open input/servers.json.2. Ensure govtIp1, govtPort1, govtIp2, govtPort2, and stateEnable fields are populated. |
| Login Packet Not Capturing in Tab 4 | Device SIM card has no network signal or GSM registration pending. | 1. Verify cellular antenna is connected.2. Ensure SIM card has active 4G data subscription.3. Wait 30-60 seconds for GSM network registration. |

---

## 9.0 OPERATOR CHECKLIST & BEST PRACTICES

### 9.1 Daily Pre-Testing Checklist

[ ] DC Power Supply set to 12V or 24V DC (2A minimum).

[ ] USB-to-Serial Debug Cable connected firmly to PC and DUT.

[ ] Cellular (4G) and GNSS (GPS) antennas connected.

[ ] Close external serial software (TeraTerm, PuTTY).

[ ] Selected physical COM port (COMx [Serial]) in FOTA Utility.

### 9.2 Operational Do's and Don'ts

DO verify your Target State dropdown selection before clicking Start Engine.

DO export CSV reports at the end of each testing shift.

DON'T disconnect serial cable or power off DUT during active FOTA download (Stage S5).

DON'T select Bluetooth serial ports.

---

## 10.0 DOCUMENT REVISION LOG

| Revision | Date | Section Changed | Description of Changes | Approved By |
| --- | --- | --- | --- | --- |
| 1.0.0 | 15-01-2026 | All | Initial release for manual serial testing. | QA Lead |
| 1.5.0 | 10-05-2026 | Sec 4, 5 | Added 3-status logic engine and automated REST polling. | QA Manager |
| 2.0.0 | 24-08-2026 | All | Complete user-focused rewrite: 10-Stage Progression Cards guide, Bluetooth guard rules, troubleshooting self-help table, and AEPL corporate PDF header. | Engineering Lead |

---

## 11.0 PREPARATION, REVIEW & APPROVAL SIGN-OFF

Action / Designation

Name

Signature

Date

PREPARED BY:Lead QA Automation Engineer

Suraj Bhalerao

____________________

24 / 08 / 2026

CHECKED BY:Embedded Hardware Manager

Accolade R&D Team

____________________

24 / 08 / 2026

APPROVED BY:Quality Assurance Director

AEPL Quality Management

____________________

24 / 08 / 2026